FPGA Software Interface

This document serves to further explain the FPGA register map.  The hardware registers listed in the register map will appear at the base of FPGA address space.

The registers marked "stock" are unchanged from the sample FPGA load.  Thus, any sample software provided that uses them should still work.

Registers marked "Custom" are specific to our application.  Here is some added detail on those:

R_BR_*:  After setting R_BR_ADDR, you can read or write a 32 bit word of data via the read or write register, respectively.

R_WPU_CTRL:  This has the following control bits:

Bit 0: 1 = enable 25MHz clock, 0 = disable

Bit 1: 1 = reset waveform processor, 0 = allow waveform processor to run

Bit 2: 1 = return test pattern for ADC data, 0 = actually read ADCs

Bit 3: 1 = reset the read side of the FIFO (for new image or image re-read)

Bit 4: 1 = reset the write side of the FIFO (for new image)

Bits 31-5: reserved

R_WPU_COUNT:  This indicates the number of times your waveform will be output before the waveform processor goes into an idle state.

R_WPU_START_STOP:  Supply 16 bit start and stop addresses in SRAM for the waveform program.

R_WPU_STATUS: Number of waveform iterations remaining

The following is the intended usage algorithm for manipulating the CCD control signals on this hardware.  Note that most of the steps must be executed on all eight units.

1. Assert CTRL bit 1 to hold waveform processor in reset. Also assert CTRL bits 3 and 4 to reset DRAM address, if desired.

2. Set COUNT and LEN as desired, and fill blockram with waveform data.

3. Assert CTRL bit 0 to turn on clock, if not already on.

4. Turn off clock.

5. Deassert CTRL bits 1, 3, 4.

6. Turn on clock.  It is this starting of the clock that makes all units work simultaneously.

7. Monitor STATUS until it is zero, which indicates the logic is done sending waveforms.  At the same time, if ADC data is being collected, monitor R_DDR_COUNT and begin reading image data from R_DDR_RD_DATA.

8. Note that it may be necessary to write as many as 256 filler words into R_DDR_WR_DATA in order to cause all the read data to come through the FIFO.