Notes on BEE FPGA Block Diagram

- The boilerplate codebase provided by RTD already includes the Xilinx PCIe block, the PCIe TLP to PIO engine, the register block, and the Xilinx memory interface.

- Red arrows indicate a 32 bit bus inside the FPGA.  Arrow direction indicates control, not necessarily data flow direction.

- The part of this that is novel is the CCD waveform processor.  This will be a custom logic block that has almost infinite flexibility in how it controls the CCDs.  Its behavior is informed by a small binary file in a format we will define.

- The 128MB SDRAM on this board is enough to hold several images.  This greatly simplifies the design.

- PCI Express is a packet-based protocol.  The Xilinx IP turns the 2.5GHz differential serial of PCIe into a 62.5MHz 32 bit bus that FPGA fabric can deal with, but it is still in packets.  Then the transaction layer packet (TLP) engine provided by RTD interprets these packets and reads and writes registers in the register block.

- The register block structure used by RTD is a very limited way of using PCI Express, but it is good enough for our purposes.  By default it supports 32 registers, each 32 bits, that the CPU board can access with simple read and write bus cycles.  Based on the project goals at this time, a more sophisticated bridge is not necessary.

Data Flow in the Image Acquisition Process

1. Software loads the desired CCD waveform binary into blockram.

2. Software writes a control register to initiate an image acquisition sequence.

3. The CCD waveform processor emits a control sequence.

4. The ADC input deserializer sees data coming in from the ADC.  It places it in the SDRAM.

5. Steps 3 and 4 are repeated (approximately 500,000 times?) until the image is complete.

6. As the image is being written to SDRAM, software can retrieve it row by row.

Not Shown in the Diagram

- The 100MHz region is clocked by an LVDS input clock so that all 8 BEEs are synchronized.  There is also logic to emit this 100MHz clock and possibly a synchronous trigger signal.

- Before data is placed in SDRAM, we plan to calculate a per-row CRC.  Software can check this CRC later after the image is safely on a disk.